Week 3 - Data Communication

Name : Swayam Rudrawar

# Objective

To enable device-to-device and device-to-cloud communication using ESP32, ThingSpeak and MQTT.

# Project 1: Device-to-Cloud Communication

DHT22 sensor data was transmitted from ESP32 to the ThingSpeak cloud platform using HTTP requests over Wi-Fi.

Architecture: DHT22 → ESP32 → Wi-Fi → ThingSpeak Cloud Dashboard

| DHT22 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO15 |

Code:-

#include <WiFi.h>

#include <HTTPClient.h>

#include "DHTesp.h"

const char* ssid = "Wokwi-GUEST";

const char* password = "";

const char* apiKey = "GVFPDOANEIGYQK3C";

DHTesp dht;

void setup() {

Serial.begin(115200);

dht.setup(15, DHTesp::DHT22);

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected");

}

void loop() {

TempAndHumidity data = dht.getTempAndHumidity();

float temp = data.temperature;

float hum = data.humidity;

Serial.print("Temperature: ");

Serial.println(temp);

Serial.print("Humidity: ");

Serial.println(hum);

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url =

"https://api.thingspeak.com/update?api_key=" +

String(apiKey) +

"&field1=" +

String(temp) +

"&field2=" +

String(hum);

http.begin(url);

int responseCode = http.GET();

Serial.print("Response Code: ");

Serial.println(responseCode);

http.end();

}

delay(20000);

}

Output:-

# Project 2: Device-to-Device Communication

MQTT communication was implemented using an ESP32 publisher and HiveMQ MQTT broker.

Architecture: ESP32 Publisher → MQTT Broker → Subscriber Client

| Topic | Purpose |
| --- | --- |
| swayam/arnita/temperature | Temperature Data |
| swayam/arnita/humidity | Humidity Data |

Code:-

#include <WiFi.h>

#include <PubSubClient.h>

#include <DHT.h>

#define DHTPIN 15

#define DHTTYPE DHT22

const char* ssid = "Wokwi-GUEST";

const char* password = "";

// MQTT Broker

const char* mqtt_server = "broker.hivemq.com";

WiFiClient espClient;

PubSubClient client(espClient);

DHT dht(DHTPIN, DHTTYPE);

void reconnect() {

Serial.println("Inside reconnect()");

while (!client.connected()) {

Serial.print("Connecting to MQTT...");

if (client.connect("ESP32_Publisher")) {

Serial.println(" Connected!");

} else {

Serial.print(" Failed, rc=");

Serial.println(client.state());

delay(2000);

}

}

}

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("================================");

Serial.println("MQTT Publisher Started");

Serial.println("================================");

dht.begin();

Serial.println("DHT22 Initialized");

Serial.print("Connecting to WiFi");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

client.setServer(mqtt_server, 1883);

Serial.println("MQTT Server Configured");

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("DHT Read Failed!");

} else {

String tempStr = String(temperature, 2);

String humStr = String(humidity, 2);

bool tempStatus =

client.publish("swayam/arnita/temperature",

tempStr.c_str());

bool humStatus =

client.publish("swayam/arnita/humidity",

humStr.c_str());

Serial.println("--------------------------------");

Serial.print("Temperature: ");

Serial.println(tempStr);

Serial.print("Humidity: ");

Serial.println(humStr);

Serial.print("Temperature Publish Status: ");

Serial.println(tempStatus);

Serial.print("Humidity Publish Status: ");

Serial.println(humStatus);

}

delay(5000);

}

Output:-

# Communication Protocol Comparison

| Protocol | Type | Speed | Internet | Application |
| --- | --- | --- | --- | --- |
| HTTP | Client-Server | Moderate | Yes | ThingSpeak |
| MQTT | Publish-Subscribe | High | Yes | Device Communication |
| Bluetooth | Peer-to-Peer | Moderate | No | Short Range Communication |

# Observations

ESP32 successfully connected to Wi-Fi.

ThingSpeak displayed live temperature and humidity data.

MQTT enabled real-time communication between publisher and subscriber.

Data transmission was reliable and continuous.

# Conclusion

Week 3 objectives were successfully achieved using ThingSpeak and MQTT communication.