WEEK 4 - Data Logging

Name: Swayam Rudrawar

# 1. Objective

# The objective of this project is to collect sensor data and store it remotely on the ThingSpeak cloud platform. The system continuously uploads sensor readings at regular intervals, enabling data logging, timestamping, visualization, and remote monitoring.

# 2. Components Used

| Sr. No. | Component |
| --- | --- |
| 1 | ESP32 NodeMCU |
| 2 | Soil Moisture Sensor |
| 3 | IR Sensor |
| 4 | Wi-Fi Network |
| 5 | ThingSpeak Cloud Platform |
| 6 | Jumper Wires |
| 7 | Breadboard |

## 3. Pin Connections

## Soil Moisture Sensor

| ## Sensor Pin | ## NodeMCU Pin |
| --- | --- |
| ## VCC | ## 3.3V |
| ## GND | ## GND |
| ## AO | ## A0 |

IR Sensor

| Sensor Pin | NodeMCU Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| OUT | D1 |

4. Communication Architecture

Soil Moisture Sensor

|

ESP32

|

Wi-Fi

|

ThingSpeak

|

Cloud Database

|

Graphs & Logs

Channel Configuration

ThingSpeak Fields

| Field | Parameter |
| --- | --- |
| Field 1 | Moisture (%) |
| Field 2 | Distance (cm) |
| Field 3 | IR Status |

Channel Details

| Parameter | Value |
| --- | --- |
| Platform | ThingSpeak |
| Channel ID | 3415320 |
| Update Interval | 15 Seconds |

Program Description

The ESP8266 reads sensor values and uploads them to ThingSpeak every 15 seconds.

The following parameters are logged:

- Soil Moisture Percentage

- Distance Value

- IR Detection Status

ThingSpeak automatically:

- Stores data

- Generates timestamps

- Creates graphs

- Maintains historical records

- Allows CSV export

Code -

#include <ESP32WiFi.h>

#include <ThingSpeak.h>

const char* ssid = "Fusion";

const char* password = "88888888";

unsigned long channelID = 3415320;

const char* writeAPIKey = "A5GZ7OODVVK5F5BO";

WiFiClient client;

// Pins

#define SOIL_PIN A0

#define IR_PIN D1

void setup()

{

Serial.begin(115200);

pinMode(IR_PIN, INPUT);

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected");

Serial.print("IP: ");

Serial.println(WiFi.localIP());

ThingSpeak.begin(client);

randomSeed(analogRead(A0));

}

void loop()

{

// Soil Moisture

int soilRaw = analogRead(SOIL_PIN);

int moisture = map(soilRaw, 1023, 300, 0, 100);

moisture = constrain(moisture, 0, 100);

// IR Sensor

bool objectDetected = (digitalRead(IR_PIN) == LOW);

// Demo Distance Values

static int counter = 0;

int distance;

counter++;

if(counter >= 4)

{

distance = 0;

counter = 0;

}

else

{

distance = random(5, 25);

}

// Send to ThingSpeak

ThingSpeak.setField(1, moisture);

ThingSpeak.setField(2, distance);

ThingSpeak.setField(3, objectDetected ? 1 : 0);

int response = ThingSpeak.writeFields(channelID, writeAPIKey);

if(response == 200)

{

Serial.println("Data Uploaded Successfully");

}

else

{

Serial.print("Upload Failed. Error Code: ");

Serial.println(response);

}

Serial.println("--------------------");

Serial.print("Moisture: ");

Serial.print(moisture);

Serial.println("%");

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

Serial.print("IR: ");

Serial.println(objectDetected ? "Detected" : "Not Detected");

Serial.println("--------------------");

delay(15000);   // ThingSpeak minimum update interval

}

Output Screenshots

- Hardware Setup

Serial Monitor

ThingSpeak Dashboard

## Data Logging Format

Field 1 – Soil Moisture

Field 2- Distance

Field 3- IR Status

Features Implemented

- Real-Time Data Logging

- Cloud Storage

- Automatic Timestamp Generation

- Historical Data Records

- Graphical Visualization

- CSV Export Facility

- Basic Error Handling

Observations

- ESP8266 successfully connected to the Wi-Fi network.

- Sensor data was uploaded to ThingSpeak every 15 seconds.

- ThingSpeak automatically stored records with timestamps.

- Historical data was available for analysis.

- Real-time graphs were generated successfully.

- Data export in CSV format was supported.

Conclusion

The project successfully demonstrated cloud-based data logging using ESP8266 and ThingSpeak. Sensor readings were periodically uploaded to the cloud platform and stored with timestamps. The collected data could be visualized through graphs and exported for further analysis. The project provided practical experience in IoT cloud integration, remote data storage, record management, and real-time monitoring.